# 10-State INS/GNSS Dead-Reckoning System – Complete Documentation

# 1. Overview

This document describes the original dead-reckoning implementation used in dr_dead_reckoning_app.c. The system fuses an MPU6050 IMU and Neo-6M GNSS receiver using two coupled Extended Kalman Filters: a quaternion-based attitude EKF and a position–velocity EKF. The total number of estimated states is ten.

# 2. System Architecture

The application runs as a multi-threaded embedded Linux program:
• IMU Thread (≈100 Hz): Reads accelerometer and gyroscope data.
• GNSS Thread (1 Hz): Reads GNSS fixes via ioctl.
• Fusion Thread: Performs EKF prediction and update cycles.

Navigation output is provided in the local ENU frame after initialization.

# 3. Coordinate Frames

• Body Frame (b): MPU6050 sensor frame.
• Navigation Frame (n): Local ENU frame.
• Earth Frame (ECEF): Used internally for GNSS conversion.

The ENU origin is set using the first valid GNSS fix. The EKF does not propagate position before this initialization.

# 4. EKF State Definition (10-State)

The system uses two EKFs:

Attitude EKF (4 states):
q = [q_w, q_x, q_y, q_z]

Position–Velocity EKF (6 states):
x = [E, N, U, vE, vN, vU]ᵀ

Total estimated states: 10

Sensor biases are not estimated explicitly.

# 5. Attitude EKF Prediction (ekf4_predict)

The attitude EKF propagates orientation using gyroscope measurements.
Quaternion kinematics are used:
q̇ = 0.5 · q ⊗ ω

After integration, the quaternion is normalized. Covariance is increased using a small process-noise model to account for gyro noise.

# 6. Attitude EKF Update (ekf4_update_accel)

The accelerometer is used to correct roll and pitch by aligning the measured specific force with the gravity vector. The update assumes low dynamic motion.

Yaw is not observable from accelerometer data and is left uncorrected.

# 7. Position–Velocity EKF Prediction (pv6_predict)

Inertial dead-reckoning is performed by rotating body-frame acceleration into ENU, subtracting gravity, and integrating to obtain velocity and position.

During GNSS outages, process noise is inflated to reflect growing uncertainty.

# 8. GNSS Position Update (pv6_update_pos)

GNSS latitude, longitude, and altitude are converted to ENU coordinates and used to update the position states directly. Measurement covariance is scaled using HDOP, and updates are gated using a chi-square test.

# 9. GNSS Velocity Update (pv6_update_vel)

GNSS speed (and optionally heading) is fused to correct velocity states. Velocity updates significantly reduce drift and improve yaw indirectly.

# 10. Non-Holonomic Constraint (pv6_update_nhc)

Ground vehicle motion constraints enforce zero lateral and vertical velocity in the body frame. This pseudo-measurement reduces lateral drift and improves yaw stability during GNSS outages.

# 11. Zero-Velocity Update (pv6_update_zupt)

When the vehicle is stationary, a zero-velocity update forces velocity states to zero. ZUPT resets accumulated inertial drift and stabilizes position.

# 12. GNSS Outage Handling

During GNSS loss:
• GNSS updates are disabled.
• Position and velocity are propagated using IMU only.
• Process noise is increased.
• NHC and ZUPT remain active.

This enables short-term dead-reckoning but drift accumulates over time.

# 13. Limitations of the 10-State Approach

• No accelerometer or gyro bias estimation.
• Rapid drift during long GNSS outages.
• Yaw remains weakly observable.
• Not suitable for extended GNSS-denied operation.

# 14. Comparison to 15-State INS EKF

Compared to the upgraded 15-state INS EKF, the 10-state design:
• Has fewer states and simpler math.
• Cannot estimate biases.
• Experiences significantly higher drift.

The 15-state error-state EKF is recommended for production systems.

# 15. Conclusion

The 10-state EKF implementation provides a functional baseline dead-reckoning solution suitable for short GNSS outages. However, its limitations motivate the transition to a 15-state INS EKF for robust, long-duration navigation.