# 10-State INS/GNSS Dead-Reckoning System – Extended Technical Documentation

# 1. Introduction

This document extends the original 10-state EKF documentation by providing detailed mathematical equations and function-level explanations corresponding directly to the implementation in dr_dead_reckoning_app.c.

# 2. EKF State Definition (10 States)

The system estimates a total of 10 states using two EKFs:

Attitude EKF (4 states):
q = [q_w, q_x, q_y, q_z]

Position–Velocity EKF (6 states):
x = [E, N, U, v_E, v_N, v_U]ᵀ

No sensor biases are estimated in this formulation.

# 3. Attitude EKF – Prediction (ekf4_predict)

Gyroscope measurements ω = [ω_x, ω_y, ω_z]ᵀ are used to propagate attitude:

q̇ = 0.5 · Ω(ω) · q

Discrete propagation:
qₖ₊₁ ≈ qₖ ⊗ exp(0.5·ω·Δt)

Covariance prediction:
Pₖ₊₁ = FₖPₖFₖᵀ + Qₖ

# 4. Attitude EKF – Accelerometer Update (ekf4_update_accel)

When linear acceleration is small, the accelerometer measures gravity:

z_a ≈ R(q)ᵀ · g

Only roll and pitch are corrected; yaw remains unobservable.

# 5. PV EKF – Prediction (pv6_predict)

Acceleration is rotated into ENU and gravity is removed:

a_n = R(q) · a_b − g

Velocity update:
vₖ₊₁ = vₖ + a_n·Δt

Position update:
pₖ₊₁ = pₖ + vₖ·Δt + 0.5·a_n·Δt²

# 6. GNSS Position Update (pv6_update_pos)

Measurement model:
z_p = H_p x + v_p,   H_p = [I₃ 0₃]

Measurement noise is scaled using HDOP² and gated via NIS.

# 7. GNSS Velocity Update (pv6_update_vel)

GNSS speed and heading form ENU velocity:

v_E = s·sin(ψ),   v_N = s·cos(ψ)

Measurement model:
z_v = H_v x + v_v,   H_v = [0₃ I₃]

# 8. Non-Holonomic Constraint (pv6_update_nhc)

Ground vehicle constraints enforce:
v_yᵇ ≈ 0, v_zᵇ ≈ 0

These are applied as pseudo-measurements.

# 9. Zero-Velocity Update (pv6_update_zupt)

When stationary:
v ≈ 0

This reduces accumulated inertial drift.

# 10. GNSS Outage Behavior

During GNSS outages, inertial prediction continues with increased process noise. NHC and ZUPT limit drift, but unmodeled biases cause faster divergence than in the 15-state EKF.

# 11. Summary

The 10-state EKF provides a functional baseline implementation but is best suited for short GNSS outages.