# 15-State INS/GNSS Dead-Reckoning System – Extended Technical Documentation

# 1. Purpose of This Document

This document extends the previous implementation documentation by providing full mathematical derivations, detailed EKF function-level flow explanations, GNSS outage and recovery flowcharts, bias convergence discussion, and a comparison between 10-state and 15-state EKF architectures.

# 2. Mathematical State Definition

Nominal State:
p = [E, N, U]ᵀ
v = [vE, vN, vU]ᵀ
q = quaternion (body → ENU)
bₐ = accelerometer bias
b_g = gyroscope bias

Error-State Vector:
δx = [δp, δv, δθ, δbₐ, δb_g]ᵀ ∈ ℝ¹⁵

# 3. Continuous-Time INS Error Dynamics

The linearized INS error-state equations are:

δṗ = δv
δṽ = −R·[f]×·δθ − R·δbₐ + wₐ
δθ̇ = −[ω]×·δθ − δb_g + w_g
δḃₐ = w_ba
δḃ_g = w_bg

Where:
R is the body-to-ENU rotation matrix
[·]× is the skew-symmetric matrix
w terms represent white Gaussian noise processes.

# 4. Discrete-Time EKF Prediction

The discrete-time state transition matrix is approximated as:

Φ ≈ I + FΔt

Covariance propagation:
Pₖ₊₁ = ΦPₖΦᵀ + Qₖ

Process noise Q is scaled during GNSS outages based on outage duration.

# 5. Measurement Models

GNSS Position:
z_p = p + v_p
H_p = [I₃ 0₃ 0₃ 0₃ 0₃]

GNSS Velocity:
z_v = v + v_v
H_v = [0₃ I₃ 0₃ 0₃ 0₃]

Non-Holonomic Constraint (NHC):
z_nhc = [v_yᵇ, v_zᵇ]ᵀ ≈ 0

Zero Velocity Update (ZUPT):
z_zupt = v ≈ 0

# 6. GNSS Outage and Recovery Flow

GNSS Outage Detection:
If (current_time − last_gnss_time) > timeout → GNSS outage

Outage Mode:
• Disable GNSS updates
• Inflate process noise Q
• Continue INS prediction
• Apply NHC + ZUPT

GNSS Recovery:
• Gate GNSS using NIS
• Fade-in measurements over multiple updates
• Reset outage timer

ASCII Flowchart:

[GNSS OK]
     ↓
[INS Predict]
     ↓
[GNSS Update?]──No──▶[Outage Mode]
     │Yes                 │
     ▼                    ▼
[Gate + Fuse]        [Q Inflate]
     │                    │
     └──────▶[Continue Navigation]

# 7. Bias Convergence Explanation

Accelerometer and gyroscope biases are modeled as random walks.
During GNSS availability, position and velocity residuals excite bias states.
The EKF gradually adjusts bₐ and b_g to minimize innovation.

During GNSS outages, bias estimates are held and slowly evolve via process noise.
Accurate bias estimation before an outage significantly reduces drift.

# 8. Conceptual Bias Convergence Plots

Typical behavior:
• Bias estimates converge exponentially during GNSS availability.
• Covariance of biases shrinks over time.
• During outages, bias covariance grows slowly.

These plots are visible by logging bₐ and b_g from the CSV output.

# 9. 10-State vs 15-State EKF Comparison

10-State EKF:
• States: p, v, q
• No bias estimation
• Faster drift during GNSS outages
• Simpler but less robust

15-State EKF:
• Adds accelerometer and gyro biases
• Much slower drift during outages
• Industry-standard INS architecture
• Required for long-duration dead reckoning

# 10. Function-Level Flow Explanation

ins15_predict():
• Bias removal
• Attitude integration
• Acceleration rotation + gravity removal
• Position/velocity propagation
• Covariance prediction

ins15_update_gnss_pos():
• Compute innovation
• NIS gating
• Correct p, v, q, biases

ins15_update_gnss_vel():
• Direct velocity correction
• Strong yaw observability improvement

ins15_update_nhc():
• Enforces ground vehicle constraints

ins15_update_zupt():
• Resets velocity drift during stops

# 11. Conclusion

The 15-state INS EKF implementation represents a professional-grade dead-reckoning system. Through rigorous modeling, physical constraints, and robust GNSS outage handling, it significantly outperforms simpler EKF designs and forms a strong foundation for future navigation system extensions.