# 15-State INS/GNSS Dead-Reckoning System – Complete Documentation

# 1. Overview

This document describes the current implementation of a 15-state Inertial Navigation System (INS) with GNSS aiding, designed for ground vehicle dead-reckoning using a Raspberry Pi 3 Model B, MPU6050 IMU, and u-blox Neo-6M GNSS receiver. The system is implemented as a production-grade, multi-threaded C application using an Error-State Extended Kalman Filter (ES-EKF).

# 2. System Architecture

The system consists of three real-time threads:
• IMU Thread (100 Hz): Reads raw accelerometer and gyroscope samples.
• GNSS Thread (1 Hz): Reads GNSS fixes via ioctl from a kernel driver.
• Fusion Thread (100 Hz): Runs INS mechanization, EKF prediction/update, GNSS outage handling, and logging.

All navigation outputs are expressed in the local ENU (East-North-Up) frame.

# 3. Coordinate Frames

• Body Frame (b): IMU sensor frame.
• Navigation Frame (n): Local-level ENU frame.
• Earth Frame (ECEF): Used internally for GNSS conversion.

The ENU origin is initialized from the first valid GNSS fix. Navigation does not start until this reference is established.

# 4. INS State Definition

The system uses a 15-state Error-State EKF.

Nominal State (propagated continuously):
• Position p = [E, N, U] (m)
• Velocity v = [vE, vN, vU] (m/s)
• Attitude q (unit quaternion, body → ENU)
• Accelerometer bias ba (m/s²)
• Gyroscope bias bg (rad/s)

Error-State Vector:
δx = [δp(3), δv(3), δθ(3), δba(3), δbg(3)]ᵀ

# 5. INS Prediction (Mechanization)

At each IMU timestep:
1. Bias-corrected gyro measurements are integrated to propagate attitude.
2. Bias-corrected accelerations are rotated into ENU and gravity is removed.
3. Velocity and position are updated using Newtonian kinematics.

The EKF covariance is propagated using a linearized state-transition matrix and discrete process noise. During GNSS outages, the process noise is inflated using outage-duration-based scaling.

# 6. GNSS Measurement Updates

When GNSS is available, the following measurements are fused:

• Position Update:
  z_p = p_ENU + noise
  Measurement covariance is scaled using HDOP².

• Velocity Update (optional):
  GNSS speed and course are converted to ENU velocity and fused directly.

Each update is gated using the Normalized Innovation Squared (NIS) test.

# 7. Non-Holonomic Constraint (NHC)

Ground vehicles cannot move laterally or vertically. This physical constraint is applied as a pseudo-measurement enforcing:
v_y^body ≈ 0 and v_z^body ≈ 0.

NHC significantly improves yaw observability and reduces lateral drift, especially during GNSS outages.

# 8. Zero-Velocity Update (ZUPT)

When the vehicle is detected as stationary (low acceleration deviation and low angular rate), a Zero-Velocity Update is applied:
v_ENU ≈ 0.

ZUPT resets accumulated velocity drift and indirectly corrects attitude and sensor biases.

# 9. GNSS Outage Handling

During GNSS loss:
• No last-fix reuse is performed.
• INS prediction continues using IMU only.
• Process noise is inflated based on outage duration.
• NHC and ZUPT remain active.
• Velocity decay is applied when nearly stationary.

When GNSS returns, measurements are reintroduced gradually using fade-in logic to prevent solution jumps.

# 10. Logging and Diagnostics

A new CSV log file is created on each application run. At 1 Hz, the following are logged:
• Timestamp
• ENU position, velocity, acceleration
• Attitude yaw
• Estimated accelerometer and gyroscope biases
• Raw IMU readings
• Raw GNSS data (lat, lon, alt, speed, heading, HDOP)
• GNSS outage duration
• Q-scale factor
• NIS values for position and velocity updates

The CSV format allows direct import into Excel for analysis.

# 11. Limitations

• Flat-Earth ENU assumption limits long-distance accuracy.
• No wheel odometry or magnetometer aiding.
• GNSS heading accuracy is limited at low speeds.
• Bias convergence depends on GNSS availability.

# 12. Future Improvements

• Add wheel odometry fusion.
• Add magnetometer aiding for yaw stabilization.
• Upgrade to tightly-coupled GNSS (raw pseudorange/Doppler).
• Implement NEES consistency testing.
• Add map-matching constraints.

# 13. Conclusion

The current 15-state INS EKF implementation provides a robust, industry-standard dead-reckoning solution using low-cost sensors. By combining bias estimation, physical constraints, adaptive noise modeling, and careful GNSS outage handling, the system achieves reliable navigation performance suitable for ground vehicles.